KC2W_V1.3.5_251216

- Resolve the issue where TCP ports cannot be used when AP-STA mode coexists

KC2W_V1.3.4_251104

- Fixed the issue where PGN127250 could not be converted to HDT at 0°.

- Resolved the problem of N2K devices being lost after upgrading from a lower version to a higher version.

KC2W_V1.3.2_250813

- Resolve the issue where the N2K function of KC-2W fails due to data being sent too quickly at 4800 baud rate.

- Add conversion between 0183 sentence TTM and PGN128520.

- Fix the issue of incomplete satellite number output in PGN129540.

- Optimize webpage refresh to resolve the slow loading of pages.

KC2W_V1.2.6_250402:

- Limited the number of WiFi connections to a maximum of two.

- Fixed web configuration bug for setting UDP unicast IP.

- Fixed failure in converting N2K indoor humidity data to 0183 format.

- Added TCP reception feature.

- Added UDP send/receive features.

- Serial received 0183 data now also supports UDP forwarding.

- Added user-configurable UDP/TCP switching.

- Added user-configurable UDP/TCP port settings.

- Optimized some web content display issues.

- Added feature allowing WiFi password to be left empty.

- Optimized hotspot name display; removed "_ESP32" from default name.

- Fixed bug where both web ports could be set to the same value.

- Added domain name feature: kc2w.local.

KC2W_V1.1.9_241126:

- Modified serial number writing logic to fix initial write failure.

- Fixed hotspot name garbled issue on the web after writing the serial number.

- Changed default WiFi config from initial password to last modified state of older versions.

- Fixed issue with hotspot name setting error on web.

- Fixed compatibility issues between new and old versions.

- Added XDR humidity handling in PGN130311.

- Added header identifier to 0183 humidity-related sentences to distinguish external vs internal humidity data.

KC2W_V1.0.0_241015:

- Changed device name to "NMEA2000<>0183 Gateway(KC-2W)"

- Set max length of WiFi name and password to 32 characters

- Fixed an issue with the compiler tool used since V0.5.3 that prevented GSA and GSV output

Version: V0.7.1

- Date: 2024-08-23

- Changes:

- Fixed incorrect device name

Version: V0.7.0

- Date: 2024-08-21

- Changes:

- Fixed ETA time issue in 129284

- Fixed bearing issue in 129284

- New feature: After each SPIFFS write, print the result information

- Changed behavior: If serial number is "Unknown_SN" after boot, and runs for 1 hour, it will auto-set UID as fallback serial number and save it permanently unless "re-burn" is performed

- Optimization: Added checksum verification for received 0183 sentences